| Payette National Forest Krassel Ranger District Fire Effects Monitoring Report |  |
| --- | --- |

Unit:	Bald Hill RX Unit F and G                                         Date: April 16 – 18, 2021

Fuel Type: Ponderosa Pine Litter

Fuel Model: TL 8

Acres Planned: 4000

Acres Burned: 1800

FEMO: Heather Heward, Christine Droske

FEMO (t): Steven Becker

Fire Effects and Meeting Objectives:

Fire behavior remained low throughout the burn period with rates of spread around 3 ch/hr and flame lengths 1-3 feet. Handn ignition was used along property lines, unit boundaries and RCAs. Aerial ignition by PSD was used for the rest of the unit starting at snow line and progressively working down slope. Resource and prescribed fire objectives were met by the burn.

Purpose and Resource Management Goals:

- Manage vegetation to meet desired future conditions as specified in the Forest Plan.

  - Promote growth of medium sized trees to large size class.

  - Reduce canopy closure in areas that have existing high canopy closures.

- Manage fire within its historic fire return interval.

- Prevent active ignition in RCAs (300’ perennial, 150’ intermittent), but allow fire to back into these areas

Resources

- Overhead 4

- Femos/ trainees 3

- Helicopter 1

- Type 4 Engines 2 (6 total personnel between the 2)

- BLM handcrew 4

- Krassel Helitack 4

- Boise handcrew 6

- Nez Perce handcrew 3

- UTV

- ATV

- Additional 4 helitack at helispot 2 HECM / 2 PLDO

Krassel Helitack 4 and one of the Type 4 engines were identified as contingency and were on site due to the long response time to the burn.

Specific Prescribed Fire Objectives:

| Objective | Obtained |  |
| --- | --- | --- |
| Fire in 40 – 80 % of target area | ~80% of target area burned |  |
| Mortality in trees >20 in. dbh <10% | Achieved |  |
| Retain 4 -10 tons/acre of material greater than 3” in diameter. | Achieved |  |
| Minimize consumption of fuels over 15” in diameter | Achieved |  |
| Maintain an average of 50% of the duff | Achieved (see table 1) |  |
| Limit fire in PVG 6 -11 (grand fir, subalpine fir, lodgepole pine) to less than 20% of its extent. | Achieved |  |

Total loading reduction on the site

Site Description:

The burn unit is on a 20 to 70% slope with south aspect. This is a second RX entry in primarily uneven-age stand of ponderosa pines between 5-20” dbh with patches of Douglas-fir. The understory is mostly pine litter.

Observed fire behavior.

Fire behavior observatiosn were taken throughout the day on each burn. The general fire behavior was very similar each day

Pine litter

Head

FL – 1 to 3 ft

ROS – 2 to 4, maxing at 6 ch/h

Back

FL – 0.5 to 1 ft

ROS – 0.5 to 1 ch/h

Low shrub (Oregon grape, kinikinick)

Head

FL – 0.5 to 1 ft

ROS – <0.5 ch/h

April 16th, 2021 (South portion of unit F)

0830	Briefing at the Whiskey Creek Parking

1120	Test fire east of whiskey creek

1900	Complete ignition

April 17th, 2021 (Unit F)

0830	Briefing at Krassel

1108	Test fire through aerial ignition

1200	First pass with aerial ignition consuming. Fuels receptive, fire working through rocks smoldering and creeping. Smoke lofting up, wispy. Isolated torching.

1330	Backing fire .5 to 1 ch/hr., smoke starting to shade out the unit.

1420	Ignitions tied into DP 2

1450	Started hand ignitions from DP 4-5

1510	Paused ignitions between DP 5 and 6 due to holding concerns.

1545	Ignitions resumed

1555	Completed ignitions at DP7. Smoke is light and wispy traveling upslope and then being carried to the south with the transport winds. Fire continuing to back downhill toward road. Primarily carrying through the needle cast.

1620	Return to Krassel

April 18th, 2021 (Unit G)

0800 	Briefing at Krassel

1040	Test fire completed successfully black line ignitions begin

1050	Smoke moving west to east, pushing up hill due to terrain driven winds

1130	Aerial ignitions working with hand ignitions to get desired fire effects

1250	Hand ignitions begin on the southwest corner of G. Aerial ignitions are coming together. Slight increase in fire behavior, achieving desired results.

1340	Aerial ignitions seemed to be achieving desired results. Clean up with hand ignition along lower edges.

1730	Return to Krassel

Fire Weather

04.16.2021

| TIME | LOCATION | DRY BULB | RH | WIND SPEED | WIND DIR. | % CLOUD COVER | FINE DEAD FUEL | PROB. OF IGNITION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1000 | DP 6 | 44 | 39 | Calm |  | 40 | 8 | 40 |
| 1100 | DP 3 | 45 | 28 | 3-5 | S | 30 | 7 | 40 |
| 1200 | DP 3 | 49 | 27 | 4-6 | S SW | 20 | 6 | 50 |
| 1300 | East of DP 3 | 52 | 21 | 4-6 | S SE | 20 | 5 | 60 |
| 1400 | DP 1.5 | 55 | 18 | 1-3 | E | 0 | 4/7 | 70/40 |
| 1500 | East of DP 1.5 | 53 | 15 | 1-3 | E | 0 | 4/7 | 70/40 |
| 1600 | West of DP 2 | 55 | 15 | 1-3 | SE | 0 | 5/7 | 60/40 |
| 1700 | DP 2 | 59 | 18 | 1-3 | S | 0 | 5/7 | 60/40 |
| 1800 | DP 1.5 | 59 | 19 | 1-3 | SW | 0 | 7/9 | 40/30 |
| 1900 | Below DP 2 | 58 | 19 | Calm |  | 0 | 7/9 | 40/30 |
| 1920 | Below DP 2 | 54 |  |  |  | 0 |  |  |

04.17 .21

| TIME | LOCATION | DRY BULB | RH | WIND SPEED | WIND DIR. | % CLOUD COVER | FINE DEAD FUEL | PROB. OF IGNITION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1100 | DP 3.5 | 51 | 30 | 1-3 | w | 0 | 7/9 | 40/30 |
| 1200 | DP 3.5 | 53 | 26 | 1-3 | sw | 0 | 6/7 | 50/30 |
| 1300 | DP 4 | 57 | 22 | 3-5 | sw | 0 | 5/8 | 60/40 |
| 1400 | DP 3.5 | 59 | 21 | 1-3 | w | 0 | 5/8 | 60/40 |
| 1500 | DP 4 | 61 | 21 | 1-3 | s | 0 | 5/8 | 60/40 |
| 1600 | DP 5 | 62 | 19 | 1-3 | sw | 0 | 5/8 | 60/40 |

4.18.2021

| TIME | LOCATION | DRY BULB | RH | WIND SPEED | WIND SPEED (GUSTS) | WIND DIR. | FINE DEAD FUEL | PROB. OF IGNITION |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1000 | DP 9 | 47 | 45 | 1-3 |  | e | 10 | 30 |
| 1100 | In unit G | 54 | 38 | 1-3 |  | e | 10 | 30 |
| 1200 | In unit G | 60 | 34 | 03 |  | sw | 6/10 | 50/30 |
| 1300 | In unit G | 61 | 25 | 2-5 |  | sw | 6/9 | 50/30 |
| 1400 | DP 3.5 | 69 | 16 | 3-5 |  | sw | 4/7 | 70/40 |
| 1500 | DP12 | 68 | 16 | 3-5 | 7-10 | sw | 4/7 | 70/40 |
| 1600 | DP12 | 68 | 16 | 3-5 |  |  | 4/7 | 70/40 |
| 1700 | DP12 | 68 | 16 | 3-5 |  |  | 4/7 | 70/40 |