RX-300 Unit 15: Meeting Objectives with RX Job Aid

# Ignition TOOLS for Meeting Objectives

| Condition | Suggested tool |
| --- | --- |
| - Small to medium fire area - Values at risk near the line - Complex unit share | - Drip torch - Fusee |
| - Steep terrain interior - Flashy fuels interior - Desire to reduce fire behavior interior | - Flare launcher - Fusee - Chucker - Pyro shot |
| - Perimeter ignition for a large fire area near the road | - Terra torch |
| - Interior ignition for a large fire area | - PSD – helicopter or UAV - Helitorch |
| - Piles | - Drip torch - Chucker - Propane torch - Fire gel |

# Ignition PATTERNS for Meeting Objectives

| Pattern | Graphic | Description | Example Objective | Residence time | Fire intensity | Fuel use | Effort | Time |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Backfire |  | Lighting the windward side and letting the fire move into the wind. Used in black lining. | Reduce the thatch in the dominant grass | High | Low | Low | Low | High |
| Strip head fire |  | Continuous strips of ignition parallel to the wind at set spacing and stagger with the intent for the strips to come together. | Limit mortality in the overstory trees while increasing mortality in the understory trees | Mod/High | Mod/High | High | High | Mod |
| Dot fire |  | Dots of ignition at set intervals with the intent for the dots to come together | Reduce the intensity of a strip head fire during drier fuel conditions. | Mod | Low/Mod | Low/Mod | Mod/High | Mod |
| Pattern | Graphic | Description | Example Objective | Residence time | Fire intensity | Fuel use | Effort | Time |
| Flank fire |  | Continuous strips of ignition moving into the wind at set spacing, forming chevrons on topography. | Limit mortality in the overstory trees while increasing mortality in the understory trees AND decreasing the time to completion. | Mod | Mod | Mod | Mod | Low/Mod |
| Centerfire |  | Ignition starts from the center, often on elevated topography. Concentric circles of ignition around the center. | Increase smoke lift and limit vegetation and animal mortality | Mod/High | Low/Mod | Mod | Mod | Mod |
| Ring fire |  | Ignition around all sides of the perimeter with fire burning inward. | Increase smoke life and increase vegetation mortality | Mod | High | Low | Low | Low |
| Jack potting |  | Dots of ignition in receptive fuel without the intent for the dots to come together | Consume available fuel and fuel use on non-available areas. | Low | Mod | Low | Mod | Mod |

Ignition TECHNIQUES for Meeting Objectives

| Tool | Description | Build MORE heat | Build LESS heat |
| --- | --- | --- | --- |
| Pattern | - Strip head - Flank - Dot - Ring - Jackpot | - Strip head - Ring | - Flank - Dot |
| Number | - Number of lighters | - More lighters | - Fewer lighters |
| Spacing | - Space between lighters | - Wet? Less spacing - Dry? More spacing | - Wet? More spacing - Dry? Less spacing |
| Stagger | - The angle or time from one lighter to another | - Flatter angle or less timing between lighters | - Steeper angle or more time between lighters |
| Speed | - How fast the lighters move | - Faster lighters | - Slower lighters |
| STOP/Pause | If you are not meeting objectives after adjusting ignition – consider STOPing or pausing ignition |  |  |

LOW - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - - --HIGH

Intensity

Consumption

Fuel Use

Effort

Fire Applications to Meet Objectives